# Lab 2: Basic Matrix Multiplication

## 1. Objective

The purpose of this lab is to implement a basic dense matrix multiplication routine.

## 2. Procedure

Step 1: Download the starter code from the GitHub classroom repo.

Step 2: Edit the file <lab-directory>/main.cu to implement the following where indicated:

- Allocate device memory

- Copy host memory to device

- Copy results from device to host

- Free device memory

Step 3: Edit the file <lab-directory>/kernel.cu to initialize the thread block and kernel grid dimensions and invoke the CUDA kernel, and to implement the matrix multiplication kernel code.  Edit the Makefile to build your project.

Step 4: Compile and test your code.

cd <lab-directory>

make

./sgemm			# Uses the default matrix sizes

./sgemm <m>			# Uses square m x m matrices

./sgemm <m> <k> <n>	# Uses (m x k) and (k x n) input matrices

It is a good idea to test and debug initially with small input dimensions. Your code is expected to work for varying input dimensions – which may or may not be divisible by your block size – so don’t forget to pay attention to boundary conditions.

Step 5: Answer the following questions in a new file named <lab-directory>/answers.txt:

- How many times is each element of each input matrix loaded during the execution of the kernel?  Show work.

- What is the memory-access to floating-point computation ratio in each thread? Consider multiplication and addition as separate operations, and ignore the global memory store at the end. Only count global memory loads towards your off-chip bandwidth.  Show work.

Step 6: Submit your assignment. You need only submit to Canvas the following files:

- answers.txt

- kernel.cu

- main.cu

- A screenshot or transcript of your program’s execution.

3. Grading:

Your submission will be graded based on the following criteria.

- Functionality/knowledge: 90%

  - Correct code and output results

  - Correct usage of CUDA library calls and C extensions

  - Correct handling of boundary cases

- Answers to questions: 10%

  - Correct answer to questions in step 5

  - Sufficient work is shown

  - Neatness and clarity